Sound Card DOS Utilities

------------------------                        April. 21, 1994

[ ESSCFG.EXE ]

ESSCFG.EXE is a DOS utility program for setting or display the

IRQ channel and DMA channel of your Sound Card. This program

can override the settings of hardware jumper and let you change

the setting without turning off your system.

To use this program:

1.  Change to the directory where the Sound Card software is

installed.

2.  Type "ESSCFG" and press <ENTER>.

3.  The program will detect and display the I/O address, current

IRQ channel and DMA channel settings of the Sound Card.

4.  Follow the instruction on screen to select the function or

exit the program.

[ ESSVOL.EXE ]

ESSVOL.EXE is a command line utility program to display and change

the Sound Card's volume settings.

To use this program:

1.  Change to the directory where the Sound Card software is

installed.

2.  Type following command in DOS prompt.

ESSVOL [/?] [/v:xx] [/l:xx] [/w:xx] [/m:xx] [/c:xx] [/s:xx]

where   no option      Display all volume settings.

/?             Display this message.

/v             Change master volume.

/l             Change line volume.

/w             Change wave volume.

/m             Change mic volume.

/c             Change CD volume.

/s             Change Synthesizer volume.

xx             Volume setting, the value range is 0 - 15.

Note: no xx means 0.

Manual Correction

-----------------

Page 11 --  The table of CD-ROM Port Address should be changed as follows.

������������������������������������������Ŀ

�   A   �   B   �   CD-ROM Port Address    �

������������������������������������������Ĵ

�  OFF  �  OFF  �           300H           �

������������������������������������������Ĵ

�  OFF  �  ON   �           320H           �

������������������������������������������Ĵ

�  ON   �  OFF  �           340H           �

������������������������������������������Ĵ

�  ON   �  ON   �           360H           �

��������������������������������������������

Page 11 --  The table of CD-ROM Type Select should be changed as follows.

����������������������������������Ŀ

�       �   CD-ROM Type Select     �

����������������������������������Ĵ

�  ON   �        Panasonic         �

����������������������������������Ĵ

�  OFF  �      Sony / Mitsumi      �

������������������������������������